Final Project (Final Exam Grade)

Adam Blake Bell

April 28, 2017

Mini-Processor with 8-bit Multiplier Function

Figure 1 This figure shows my original unedited code being tested in a timing simulation like figure 7.75 proving my original code is functional. Therefore the processor works correctly before adding the function of the 8 bit multiplier.

Figure 2 This is a timing simulation of my processor code with the added 8 bit multiplier. What I attempted to do was load 2A in hexadecimal onto the register selected by Rx=01 and then load 5C on the register selected by Rx=10. I then attempted to multiply the contents of register Rx=01 and register Rx=10. However while the numbers were moved onto the bus, they were never successfully stored in the registers, so the multiplication couldn’t happen. I am unsure what is causing this to happen, it seems that my code should logically work. The timing steps are correct however and the Done signal goes high at the correct times as well. If I was able to get my data from the bus to the registers, every other function should work properly. The 8 bit multiplier circuit preforms perfectly when tested outside of the processor itself.

Figure 3 This is another example of my code failing to load data onto the register. I cannot seem to find what is wrong with my code. I believed everything was in place for the multiply function to work along with all the original functions.

VHDL files used used to build the original/unedited processor are listed below.

--Fig. 7.61.. Code for an n-bit register of the type in Figure 7.56. currently an 8 bit

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY regn IS

GENERIC ( N : INTEGER := 8);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END regn ;

ARCHITECTURE Behavior OF regn IS

BEGIN

PROCESS

BEGIN

WAIT UNTIL Clock'EVENT AND Clock= '1' ;

IF Rin= '1' THEN

Q <= R;

END IF ;

END PROCESS ;

END Behavior ;

--figure 7.72,  two-bit up-counter with synchronous reset.

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE ieee.std_logic_unsigned.all ;

ENTITY upcount IS

PORT ( Clear, Clock : IN STD_LOGIC ;

Q : BUFFER STD_LOGIC_VECTOR(1 DOWNTO 0) ) ;

END upcount ;

ARCHITECTURE Behavior OF upcount IS

BEGIN

upcount: PROCESS ( Clock )

BEGIN

IF (Clock'EVENT AND Clock= '1') THEN

IF Clear= '1' THEN

Q <= "00" ;

ELSE

Q <= Q+'1' ;

END IF ;

END IF;

END PROCESS;

END Behavior ;

--Figure 7.62 code for an n-bit tristate buffer. currently 8 bits

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY trin IS

GENERIC ( N : INTEGER := 8);

PORT ( X : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

E : IN STD_LOGIC ;

F : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0)) ;

END trin ;

ARCHITECTURE Behavior OF trin IS

BEGIN

F <= (OTHERS => 'Z') WHEN E = '0' ELSE X ;

END Behavior ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

--Figure 6.30 VHDL code for a 2-to-4 binary decoder.

ENTITY dec2to4 IS

PORT ( w : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

En : IN STD_LOGIC ;

y : OUT STD_LOGIC_VECTOR(0 TO 3) ) ;

END dec2to4 ;

ARCHITECTURE Behavior OF dec2to4 IS

SIGNAL Enw : STD_LOGIC_VECTOR(2 DOWNTO 0) ;

BEGIN

Enw <= En & w ;

WITH Enw SELECT

y <= "1000" WHEN "100",

"0100" WHEN "101",

"0010" WHEN "110",

"0001" WHEN "111",

"0000" WHEN OTHERS ;

END Behavior ;

--modified figure 7.64

--package used in processor

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

PACKAGE subccts IS

COMPONENT regn -- register

GENERIC ( N : INTEGER := 8);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END COMPONENT ;

COMPONENT dec2to4

PORT ( w : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

En : IN STD_LOGIC ;

y : OUT STD_LOGIC_VECTOR(0 TO 3) ) ;

END COMPONENT ;

COMPONENT trin -- tri-state buffers

GENERIC ( N : INTEGER := 8);

PORT ( X : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

E : IN STD_LOGIC ;

F : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END COMPONENT ;

COMPONENT upcount

PORT ( Clear, Clock : IN STD_LOGIC ;

Q : BUFFER STD_LOGIC_VECTOR(1 DOWNTO 0) ) ;

END COMPONENT ;

END subccts ;

--page 462 for timing

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE ieee.std_logic_signed.all ;

USE work.subccts.all ;

--code from figure 7.74

ENTITY proc IS

PORT ( Data : IN STD_LOGIC_VECTOR(7 DOWNTO 0) ;

Reset, w : IN STD_LOGIC ;

Clock : IN STD_LOGIC ;

F, Rx, Ry : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

Done : BUFFER STD_LOGIC ;

BusWires : INOUT STD_LOGIC_VECTOR(7 DOWNTO 0) ) ;

END proc ;

ARCHITECTURE Behavior OF proc IS

SIGNAL X, Y, Rin, Rout : STD_LOGIC_VECTOR(0 TO 3) ;

SIGNAL Clear, High, AddSub : STD_LOGIC ;

SIGNAL Extern, Ain, Gin, Gout, FRin : STD_LOGIC ;

SIGNAL Count, Zero, T, I : STD_LOGIC_VECTOR(1 DOWNTO 0) ;

SIGNAL R0, R1, R2, R3 : STD_LOGIC_VECTOR(7 DOWNTO 0) ;

SIGNAL A, Sum, G : STD_LOGIC_VECTOR(7 DOWNTO 0) ;

SIGNAL Func, FuncReg, Sel : STD_LOGIC_VECTOR(1 TO 6) ;

BEGIN

Zero <= "00" ; High <= '1' ;

Clear <= Reset OR Done OR (NOT w AND NOT T(1) AND NOT T(0)) ;

counter: upcount PORT MAP ( Clear, Clock, Count ) ;

T <= Count ;

Func <= F & Rx & Ry ;

FRin <= w AND NOT T(1) AND NOT T(0) ;

functionreg: regn GENERIC MAP ( N=> 6)

PORT MAP ( Func, FRin, Clock, FuncReg ) ;

I <= FuncReg(1 TO 2) ;

decX: dec2to4 PORT MAP ( FuncReg(3 TO 4), High, X ) ;

decY: dec2to4 PORT MAP ( FuncReg(5 TO 6), High, Y ) ;

controlsignals: PROCESS ( T, I, X, Y )

BEGIN

Extern <= '0' ; Done <= '0' ; Ain <= '0' ; Gin <= '0' ;

Gout <= '0' ; AddSub <= '0' ; Rin <= "0000" ; Rout <= "0000" ;

CASE T IS	WHEN "00"=> -- no signals asserted in time step T0

WHEN "01"=> -- define signals asserted in time step T1

CASE I IS

WHEN "00"=> -- Load

Extern <= '1' ; Rin <= X ; Done <= '1' ;

WHEN "01"=> --Move

Rout <= Y ; Rin <= X ; Done <= '1' ;

WHEN OTHERS=> -- Add, Sub

Rout <= X ; Ain <= '1' ;

END CASE ;

WHEN "10"=> -- define signals asserted in time step T2

CASE I IS

WHEN "10"=> -- Add

Rout <= Y ; Gin <= '1' ;

WHEN "11"=> -- Sub

Rout <= Y ; AddSub <= '1' ; Gin <= '1' ;

WHEN OTHERS=> -- Load, Move

END CASE ;

WHEN OTHERS=> -- define signals asserted in time step T3

CASE I IS WHEN "00"=> -- Load

WHEN "01"=> --Move

WHEN OTHERS=> -- Add, Sub

Gout <= '1' ; Rin <= X ; Done <= '1' ;

END CASE ;

END CASE ;

END PROCESS ;

reg0: regn PORT MAP ( BusWires, Rin(0), Clock, R0 ) ;

reg1: regn PORT MAP ( BusWires, Rin(1), Clock, R1 ) ;

reg2: regn PORT MAP ( BusWires, Rin(2), Clock, R2 ) ;

reg3: regn PORT MAP ( BusWires, Rin(3), Clock, R3 ) ;

regA: regn PORT MAP ( BusWires, Ain, Clock, A ) ;

alu: WITH AddSub SELECT

Sum <= A+BusWires WHEN '0',

A-BusWires WHEN OTHERS ;

regG: regn PORT MAP ( Sum, Gin, Clock, G ) ;

Sel <= Rout & Gout & Extern ;

WITH Sel SELECT

BusWires <= R0 WHEN "100000",

R1 WHEN "010000",

R2 WHEN "001000",

R3 WHEN "000100",

G WHEN "000010",

Data WHEN OTHERS ;

END Behavior ;

THE EDITED CODE STARTS HERE

--page 462 for timing

--3 to 8 decoders are not necessary after consideration

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE ieee.std_logic_signed.all ;

USE work.subccts.all ;

--code from figure 7.74

ENTITY proc IS

PORT ( Data : IN STD_LOGIC_VECTOR(7 DOWNTO 0) ;

Reset, w : IN STD_LOGIC ;

Clock : IN STD_LOGIC ;

Rx, Ry : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

F : IN STD_LOGIC_VECTOR(2 DOWNTO 0) ; --changed

Done : BUFFER STD_LOGIC ;

BusWires : INOUT STD_LOGIC_VECTOR(7 DOWNTO 0);

Xwires : OUT STD_LOGIC_VECTOR(3 DOWNTO 0)) ;

END proc ;

ARCHITECTURE Behavior OF proc IS

SIGNAL X, Y, Rin, Rout : STD_LOGIC_VECTOR(0 TO 3) ;

SIGNAL Clear, High, AddSub : STD_LOGIC ;

SIGNAL Extern, Ain, Gin, Gout, FRin, Min, Pin, P7to0out, P15to8out : STD_LOGIC ;----THIS WAS ADDED Min, Pin signals

SIGNAL I, Zero, T, Count : STD_LOGIC_VECTOR(2 DOWNTO 0) ;--changed

SIGNAL R0, R1, R2, R3 : STD_LOGIC_VECTOR(7 DOWNTO 0) ;

SIGNAL A, Sum, G, M  : STD_LOGIC_VECTOR(7 DOWNTO 0) ; -- M was added

SIGNAL Sel : STD_LOGIC_VECTOR(1 TO 8) ;

SIGNAL Func, FuncReg : STD_LOGIC_VECTOR(1 TO 7) ;-- added another bit to select multiply..

SIGNAL P, D : STD_LOGIC_VECTOR(15 DOWNTO 0) ; -- output of multiply which is P and output of register P is D

BEGIN

Xwires <= X;

Zero <= "000" ; High <= '1' ; --added another bit to zero

Clear <= Reset OR Done OR (NOT w AND NOT T(2) AND NOT T(1) AND NOT T(0)) ; --COULD BE MY PROBLEM?? probably not since stepping through times in VWF

counter: upcount PORT MAP ( Clear, Clock, Count ) ;

T <= Count ;

Func <= F & Rx & Ry ;

FRin <= w AND NOT T(1) AND NOT T(0) ;

functionreg: regn GENERIC MAP ( N=> 7) --this is the function register, changed to 7 bits

PORT MAP ( Func, FRin, Clock, FuncReg ) ;

I <= FuncReg(1 TO 3); --changed because we need 3 bits of I

decX: dec2to4 PORT MAP ( Rx, High, X ) ; --changed

decY: dec2to4 PORT MAP ( Ry, High, Y ) ; --changed

controlsignals: PROCESS ( T, I, X, Y )

--The code sets T to the value of Count, while I is set to the value of the

-- three left-most bits in the Function Register, which correspond to the stored values of the input F.

BEGIN

Extern <= '0' ; Done <= '0' ; Ain <= '0' ; Gin <= '0' ; Min <= '0' ; Pin <= '0' ; --added initial Pin and Min

Gout <= '0' ; AddSub <= '0' ; Rin <= "0000" ; Rout <= "0000" ; P15to8out <= '0'; P7to0out <= '0';

--Rin and Rout select the register to release or accept data based on the encoding of X and Y

CASE T IS

WHEN "000"=> -- no signals asserted in time step T0

WHEN "001"=> -- define signals asserted in time step T1. this(originally '01') along with the rest were changed

CASE I IS

WHEN "000"=> -- Load

Extern <= '1' ; Rin <= X ; Done <= '1' ;

WHEN "001"=> --Move

Rout <= Y ; Rin <= X ; Done <= '1' ;

WHEN "010"=> -- Add, Sub

Rout <= X ; Ain <= '1' ;

WHEN "011"=> -- Add, Sub

Rout <= X ; Ain <= '1' ;

WHEN OTHERS=> -- Mult

Rout <= X ; Min <= '1' ;

END CASE ;

WHEN "010"=> -- define signals asserted in time step T2

CASE I IS

WHEN "010"=> -- Add

Rout <= Y ; Gin <= '1' ;

WHEN "011"=> -- Sub

Rout <= Y ; AddSub <= '1' ; Gin <= '1' ;

WHEN "000"=> -- Load, Move

WHEN "001"=> -- Load, Move

WHEN OTHERS=> --Mult

Rout <= Y ; Pin <= '1' ;

END CASE ;

WHEN "011"=> -- define signals asserted in time step T3

CASE I IS

WHEN "000"=> -- Load

WHEN "001"=> --Move

WHEN "010"=> -- Add, Sub

Gout <= '1' ; Rin <= X ; Done <= '1' ;

WHEN "011"=> -- Add, Sub

Gout <= '1' ; Rin <= X ; Done <= '1' ;

WHEN OTHERS=> --Mult

P15to8out <= '1'; Rin <= X;

END CASE ;

WHEN OTHERS=> -- define signals asserted in time step T4

CASE I IS

WHEN "000"=> -- Load

WHEN "001"=> --Move

WHEN "010"=> -- Add, Sub

WHEN "011"=> -- Add, Sub

WHEN OTHERS=> --Mult

P7to0out <= '1'; Rin <= Y; Done <= '1' ; --maybe just need P?

END CASE ;

END CASE ;

END PROCESS ;

reg0: regn PORT MAP ( BusWires, Rin(0), Clock, R0 ) ;

reg1: regn PORT MAP ( BusWires, Rin(1), Clock, R1 ) ;

reg2: regn PORT MAP ( BusWires, Rin(2), Clock, R2 ) ;

reg3: regn PORT MAP ( BusWires, Rin(3), Clock, R3 ) ;

regA: regn PORT MAP ( BusWires, Ain, Clock, A ) ;

alu: WITH AddSub SELECT

Sum <= A+BusWires WHEN '0', --when AddSub is zero

A-BusWires WHEN OTHERS ;

regG: regn PORT MAP ( Sum, Gin, Clock, G ) ;

regM: regn PORT MAP ( BusWires, Min, Clock, M ) ;----this added register M

multiplier: firstrow PORT MAP (M, BusWires, P); -- Multiplier added

regP: regnP PORT MAP ( P, Pin, Clock, D) ;--this is regsiter P*************** HOW DO I OUTPUT ONLY HALF

Sel <= Rout & Gout & Extern & P15to8out & P7to0out ; -- concatenate operator p. 364 in the book. Rout is 4 bits

WITH Sel SELECT

BusWires <= R0 WHEN "10000000",

R1 WHEN "01000000",

R2 WHEN "00100000",

R3 WHEN "00010000",

G WHEN "00001000",

D(15 DOWNTO 8) WHEN "00000010",--this controls when half of P goes on the bus

D(7 DOWNTO 0) WHEN "00000001",

Data WHEN OTHERS ; --others and the extern high condition

END Behavior ;

--Fig. 7.61.. Code for an 16-bit register of the type in Figure 7.56. currently a 16 bit

--This is the code for register P

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY regnP IS

GENERIC ( N : INTEGER := 16);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END regnP ;

ARCHITECTURE Behavior OF regnP IS

BEGIN

PROCESS

BEGIN

WAIT UNTIL Clock'EVENT AND Clock= '1' ;

IF Rin= '1' THEN

Q <= R;

END IF ;

END PROCESS ;

END Behavior ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE work.blockpackages_package.all ;

--this is the code that will be called for every

--row after the first row

ENTITY allotherrows IS

PORT (Cin, Q : IN STD_LOGIC;

M, PPi : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

P : OUT STD_LOGIC;

PPiout : OUT STD_LOGIC_VECTOR(7 DOWNTO 0));

END allotherrows ;

ARCHITECTURE Structure OF allotherrows IS

--the temporary signal is used for the carry ins and outs

--between the blocks in a row

SIGNAL Carry1 : STD_LOGIC_VECTOR(0 TO 6);

BEGIN

--there are 8 connected blocks in this code that are called from

--block C

secondrow2 : blockC PORT MAP (Q, Cin, M(0), PPi(0), P, Carry1(0));

secondrow3 : blockC PORT MAP (Q, Carry1(0), M(1), PPi(1), PPiout(0), Carry1(1));

secondrow4 : blockC PORT MAP (Q, Carry1(1), M(2), PPi(2), PPiout(1), Carry1(2));

secondrow5 : blockC PORT MAP (Q, Carry1(2), M(3), PPi(3), PPiout(2), Carry1(3));

secondrow6 : blockC PORT MAP (Q, Carry1(3), M(4), PPi(4), PPiout(3), Carry1(4));

secondrow7 : blockC PORT MAP (Q, Carry1(4), M(5), PPi(5), PPiout(4), Carry1(5));

secondrow8 : blockC PORT MAP (Q, Carry1(5), M(6), PPi(6), PPiout(5), Carry1(6));

secondrow9 : blockC PORT MAP (Q, Carry1(6), M(7), PPi(7), PPiout(6), PPiout(7));

END Structure;

LIBRARY work;

Library ieee ;

USE ieee.std_logic_1164.all ;

PACKAGE blockpackages_package IS

--this file is blockB and blockC packaged

COMPONENT blockB IS

PORT ( q0, q1, Cin, Mk, Mkplus1 :IN STD_LOGIC ;

s, Cout : OUT STD_LOGIC ) ;

END COMPONENT ;

COMPONENT blockC IS

PORT ( qj, Cin, Mk, PPi :IN STD_LOGIC ;

s, Cout : OUT STD_LOGIC ) ;

END COMPONENT ;

END blockpackages_package ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY blockB IS

--blockB is the code for a single block

--in the top row of the multiplier

--blockB uses two AND gates that feed into

--a Full Adder

PORT ( q0, q1, Cin, Mk, Mkplus1 :IN STD_LOGIC ;

s, Cout : OUT STD_LOGIC ) ;

END blockB ;

ARCHITECTURE LogicFunc OF blockB IS

BEGIN

s <= (Mkplus1 AND q0) XOR (Mk AND q1) XOR Cin ;

Cout <= ((Mkplus1 AND q0) AND (Mk AND q1)) OR (Cin AND (Mkplus1 AND q0)) OR (Cin AND (Mk AND q1)) ;

END LogicFunc ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY blockC IS

--this file contains the code for blockC

--this kind of block make up all the rows except for

--the first row of the 8 bit multiplier

PORT ( qj, Cin, Mk, PPi : IN STD_LOGIC;

s, Cout : OUT STD_LOGIC);

END blockC ;

--blockC uses one AND gate and a Full Adder

ARCHITECTURE LogicFunc OF blockC IS

BEGIN

s <= (PPi) XOR (Mk AND qj) XOR Cin ;

Cout <= ((PPi) AND (Mk AND qj)) OR (Cin AND (PPi)) OR (Cin AND (Mk AND qj)) ;

END LogicFunc ;

LIBRARY work;

Library ieee ;

USE ieee.std_logic_1164.all ;

PACKAGE allhardware_package IS

--this package file contains both

--blockC and blockB and the allotherrows file

COMPONENT allotherrows IS

PORT (Cin, Q : IN STD_LOGIC;

M, PPi : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

P: OUT STD_LOGIC;

PPiout : OUT STD_LOGIC_VECTOR(7 DOWNTO 0));

END COMPONENT;

COMPONENT blockB IS

PORT ( q0, q1, Cin, Mk, Mkplus1 :IN STD_LOGIC ;

s, Cout : OUT STD_LOGIC ) ;

END COMPONENT ;

COMPONENT blockC IS

PORT ( qj, Cin, Mk, PPi :IN STD_LOGIC ;

s, Cout : OUT STD_LOGIC ) ;

END COMPONENT ;

END allhardware_package ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE work.allhardware_package.all ;

--this file is named firstrow however

--it is my top level code containing

--the complete 8 bit multiplier

ENTITY firstrow IS

PORT ( M, Q : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

P : OUT STD_LOGIC_VECTOR(15 DOWNTO 0));

END firstrow ;

ARCHITECTURE Structure OF firstrow IS

--below are my temporary signals that transmit data

--suchs as partial products and carry ins and outs between

--the individual rows and blocks

SIGNAL PP1: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL PP2: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL PP3: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL PP4: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL PP5: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL PP6: STD_LOGIC_VECTOR(7 DOWNTO 0);

SIGNAL Carry0 : STD_LOGIC_VECTOR(1 TO 7);

BEGIN

P(0) <= M(0) AND Q(0);

--Above is the AND gate used on the top right of the circuit

firstrow1: blockB PORT MAP (Q(0), Q(1), '0', M(0), M(1), P(1), Carry0(1));

firstrow2: blockB PORT MAP (Q(0), Q(1), Carry0(1), M(1), M(2), PP1(0), Carry0(2));

firstrow3: blockB PORT MAP (Q(0), Q(1), Carry0(2), M(2), M(3), PP1(1), Carry0(3));

firstrow4: blockB PORT MAP (Q(0), Q(1), Carry0(3), M(3), M(4), PP1(2), Carry0(4));

firstrow5: blockB PORT MAP (Q(0), Q(1), Carry0(4), M(4), M(5), PP1(3), Carry0(5));

firstrow6: blockB PORT MAP (Q(0), Q(1), Carry0(5), M(5), M(6), PP1(4), Carry0(6));

firstrow7: blockB PORT MAP (Q(0), Q(1), Carry0(6), M(6), M(7), PP1(5), Carry0(7));

firstrow8: blockB PORT MAP (Q(0), Q(1), Carry0(7), M(7), '0', PP1(6), PP1(7));

--Above are the 8 blocks used on the top row of the multiplier. They call in my blockB code.

secondrow: allotherrows PORT MAP ('0', Q(2), M(7 DOWNTO 0), PP1(7 DOWNTO 0), P(2), PP2(7 DOWNTO 0));

--Above is the second row of blocks. It call in my allotherrows code since the second row to the seventh row are the same.

thirdrow: allotherrows PORT MAP ('0', Q(3), M(7 DOWNTO 0), PP2(7 DOWNTO 0), P(3), PP3(7 DOWNTO 0));

--Above is the third row of blocks

fourthrow: allotherrows PORT MAP ('0', Q(4), M(7 DOWNTO 0), PP3(7 DOWNTO 0), P(4), PP4(7 DOWNTO 0));

--Above is the fourth row of blocks

fifthrow: allotherrows PORT MAP ('0', Q(5), M(7 DOWNTO 0), PP4(7 DOWNTO 0), P(5), PP5(7 DOWNTO 0));

--Above is the fifth row of blocks

sixthrow: allotherrows PORT MAP ('0', Q(6), M(7 DOWNTO 0), PP5(7 DOWNTO 0), P(6), PP6(7 DOWNTO 0));

--Above is the sixth row of blocks

seventhrow: allotherrows PORT MAP ('0', Q(7), M(7 DOWNTO 0), PP6(7 DOWNTO 0), P(7), P(15 DOWNTO 8));

--Above is the seventh row of blocks

END Structure;

--modified figure 7.64

--package used in processor

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

PACKAGE subccts IS

COMPONENT regn -- register

GENERIC ( N : INTEGER := 8);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END COMPONENT ;

COMPONENT dec2to4

PORT ( w : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

En : IN STD_LOGIC ;

y : OUT STD_LOGIC_VECTOR(0 TO 3) ) ;

END COMPONENT ;

COMPONENT trin -- tri-state buffers

GENERIC ( N : INTEGER := 8);

PORT ( X : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

E : IN STD_LOGIC ;

F : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END COMPONENT ;

COMPONENT upcount

PORT ( Clear, Clock : IN STD_LOGIC ;

Q : BUFFER STD_LOGIC_VECTOR(2 DOWNTO 0) ) ;

END COMPONENT ;

--added multiplier to this

COMPONENT firstrow

PORT ( M, Q : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

P : OUT STD_LOGIC_VECTOR(15 DOWNTO 0));

END COMPONENT ;

--added 16 bit register

COMPONENT regnP

GENERIC ( N : INTEGER := 16);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END COMPONENT ;

END subccts ;

--Fig. 7.61.. Code for an n-bit register of the type in Figure 7.56. currently an 8 bit

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

ENTITY regn IS

GENERIC ( N : INTEGER := 8);

PORT ( R : IN STD_LOGIC_VECTOR(N-1 DOWNTO 0) ;

Rin, Clock : IN STD_LOGIC ;

Q : OUT STD_LOGIC_VECTOR(N-1 DOWNTO 0) ) ;

END regn ;

ARCHITECTURE Behavior OF regn IS

BEGIN

PROCESS

BEGIN

WAIT UNTIL Clock'EVENT AND Clock= '1' ;

IF Rin= '1' THEN

Q <= R;

END IF ;

END PROCESS ;

END Behavior ;

--figure 7.72,  two-bit up-counter with synchronous reset.

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

USE ieee.std_logic_unsigned.all ;

ENTITY upcount IS

PORT ( Clear, Clock : IN STD_LOGIC ;

Q : BUFFER STD_LOGIC_VECTOR(2 DOWNTO 0) ) ; --added another bit

END upcount ;

ARCHITECTURE Behavior OF upcount IS

BEGIN

upcount: PROCESS ( Clock )

BEGIN

IF (Clock'EVENT AND Clock= '1') THEN

IF Clear= '1' THEN

Q <= "000" ; --added another bit

ELSE

Q <= Q+'1' ;

END IF ;

END IF;

END PROCESS;

END Behavior ;

LIBRARY ieee ;

USE ieee.std_logic_1164.all ;

--Figure 6.30 VHDL code for a 2-to-4 binary decoder.

ENTITY dec2to4 IS

PORT ( w : IN STD_LOGIC_VECTOR(1 DOWNTO 0) ;

En : IN STD_LOGIC ;

y : OUT STD_LOGIC_VECTOR(0 TO 3) ) ;

END dec2to4 ;

ARCHITECTURE Behavior OF dec2to4 IS

SIGNAL Enw : STD_LOGIC_VECTOR(2 DOWNTO 0) ;

BEGIN

Enw <= En & w ;

WITH Enw SELECT

y <= "1000" WHEN "100",

"0100" WHEN "101",

"0010" WHEN "110", --might need to change encoder along with func register and its declaration

"0001" WHEN "111",

"0000" WHEN OTHERS ;

END Behavior ;